2MP 52x Zoom Camera Module

SOAR-CB2252

- Max Resolution: 2MP (1920×1080), Max Output: Full HD 1920×1080@30fps Live Image

- Support H.265/H.264 Video Compression Algorithm, Multi-level Video Quality Configuration and Encoding Complexity Settings

- Starlight Low Illumination, 0.0005Lux/F1.4 (Color), 0.0001Lux/F1.4 (B/W), 0 Lux with IR

- 52x Optical Zoom, 16x Digital Zoom

- Support Area Intrusion Detection, Cross-border Detection,

- Movement Detection

- Support Onvif

- ICR Automatic Switching, 24 Hours Day and Night Monitor

- Support Backlight Compensation, Automatic Electronic Shutter,

- Adapt to Different Monitoring Environment

- Support 3D Digital Noise Reduction, High light Suppression,

- Electronic Image Stabilization, 120dB Optical Width Dynamics

- Support 255 Presets, 8 Patrols

- Support Timed Capture and Event Capture

- Support One-click Watch and One-click Cruise Functions

| - Specification |  |
| --- | --- |
| Camera |  |
| Image Sensor | 1/2.8" progressive scan CMOS |
| Min. Illumination | Color: 0.0005 Lux @(F1.4, AGC ON) |
|  | Black: 0.0001 Lux @(F1.4, AGC ON) |
| Shutter Time | 1/25~1/100,000 s |
| Automatic Aperture | DC drive |
| Day& Night | ICR |
| Digital Zoom | 16x |
| Lens |  |
| Focal Length | 6.1-317mm, 52x Optical Zoom |
| Aperture Range | F1.4-F4.7 |
| Field of View | H: 61.8-1.6° (Wide - Tele) |
| Working Distance | 100mm-2000mm (Wide - Tele) |
| Zoom Speed | Approximately 6s (optional, wide-tele) |
| Compression Standard |  |
| Video Compression | H.265 / H.264 |
| H.265 encoding type | Main Profile |
| H.264 encoding type | Base Line Profile / Main Profile / High Profile |
| Video Bitrate | 32 Kbps~16Mbps |
| Audio Compression | G.711alaw/G.711ulaw/G.722.1/G.726/MP2L2/PCM |
| Audio Bitrate | 64Kbps(G.711)/16Kbps(G.722.1)/16Kbps(G.726)/32-192Kbps(MP2L2)/16-64Kbps(AAC) |
| Image |  |
| Main Stream Resolution | 50Hz: 25fps (1920 × 1080, 1280 × 960, 1280 × 720); 60Hz: 30fps (1920 × 1080, 1280 × 960, 1280 × 720) |
| Third Stream Resolution and Frame Rate | Independent of main stream settings, supports up to: 50Hz: 25fps (704 × 576); 60Hz: 30fps (704 × 576) |
| Image Setting | Corridor mode, saturation, brightness, contrast and sharpness can be adjusted by client or browser |
| Backlight Compensation | Support |
| Exposure Mode | Automatic exposure/aperture priority/shutter priority/manual exposure |
| Focus Control | Auto focus/one focus/manual focus/Semi-Auto Focus |
| Area Exposure/Focus | Support |
| Day &Night | Auto(ICR) / Color / B/W |
| 3D Noise Reduction | Support |
| Image overlay | Support BMP 24 bit image overlay, optional region |
| ROI | ROI supports one fixed region for each three-bit stream |
| Network Function |  |
| Network Storage | Built-in memory card slot, support Micro SD/SDHC/SDXC, up to 256 GB; NAS (NFS, SMB/ CIFS) |
| Protocol | TCP/IP,ICMP,HTTP,HTTPS,FTP,DHCP,DNS,RTP,RTSP,RTCP,NTP,SMTP,SNMP,IPv6 |
| Interface Protocol | ONVIF(PROFILE S,PROFILE G) |
| Smart Function |  |
| Intelligent Detection | Cross boundary detection, area intrusion detection, exit area detection, wandering detection, people gathering detection, fast motion detection |
| Interface |  |
| External interface | 36pin FFC (Including network ports、RS485、RS232、CVBS、SDHC、Alarm In/Out、Line In/Out、Power), USB |
| General |  |
| Working Environment | -30℃~60℃; Humidity less than 95% (non-condensing) |
| Power supply | DC12V±10% |
| Consumption | 2.5W MAX (IR, 5.5W MAX) |
| Dimensions | 175.5x75x78mm |
| Weight | 925g |

- Dimension